Commodore PET/CBM 1541 Adapter Rev. 0

Prototype Testing

# Test Setup

- Prototype Rev. 0 installed in the User Port

- CBM8032

- 2532 EPROM “VC-1541-DOS/80” in UD11

- Cassette Dongle with ribbon cable for power supply

- 1541 and 1541-II disk drives with different floppy disks and serial cable

- SD2IEC with a suitable disk image on SD-card

- Pi1541 with a suitable disk image on SD-card

- 1581 with a suitable disk

- A C64 setup with disk drive to check the files written with

# Test Execution

With each drive, the same tests were executed:

- Showing the disk status with !@

- Formatting the disk with !@”N:CBM8032,00

- Showing the directory with !catalog

- Saving a program with !save”<prog name>”

- Verifying the program with !verify”<prog name>”

- Loading a program with !load”<prog name>

- Writing a SEQ file with the program “seqtest”

- Reading the SEQ file

The sequence of the test steps are not necessarily the same. The program “seqtest” looks like this:

2 print"scratch file"
5 !@"s:test"
7 print"open test for writing
10 !open#1,"test,s,w"
20 fori=1to100
30 !print#1,"This is a test"
40 next
50 !close#1

## General Results

All tests were executed successfully except reading back the sequential file with the CBM8032 on any drive. It turned out, that the file number for !open, !input# etc. is rather a secondary address. An !open#15,”test,s,r” returned a “syntax error” in the status and SA 15 is for sending commands to the disk drive. An !input#2,a$ always returned a “G” in a$ (note: the software was found in an old EPROM and scope of the project is to reconstruct the hardware).

The SEQ file from the CBM8032 was the read with a program on the C64.

50 INPUT"FILE NAME";N$
70 OPEN2,8,2,N$+",S,R"
80 INPUT#2,A$
90 PRINTA$
100 IF ST<>0 THEN120
110 GOTO80
120 CLOSE2

The result looks ok.

Figure 1: The SEQ file read with a C64

Further, the SD2IEC was not capable of the !catalog command. A !load”$” did work, though.

## Pictures of the tests

Figure 2: CBM8032 with a 1541 (SpeedDOS ROM)

Figure 3: Software start, status and directory (1541)

Figure 4: Setup with the 1541-II (SpeedDOS ROM)

Figure 5: Setup with the SD2IEC

Figure 6: Software start and status (SD2IEC)

Figure 7: directory and change directory instruction (SD2IEC)

Figure 8: Formatting a disk image (SD2IEC)

Figure 9: Setup with Pi1541

Figure 10: Setup with the 1581

Figure 11: Software start and status (1581)

Figure 12: !catalog with the 1581

Figure 13: Test of the 1581 (!save, !verify, !load)

Figure 14: Setup with the Oceanic OC-118N

Figure 15: Status (OC-118N)

# Result

The hardware of the PET/CBM 1541 Adapter is fully functional. The software, that was found in the EPROM seems to work at least mostly. It is not sure, if the !input# and !get# instructions work properly, since the proper instruction syntax might not be figured out, yet.